[image: image1.jpg]BEBON

BebonChina.com

3




[image: image2.jpg]


                                   Henan BEBON international co.,ltd

Boiler steel

Boiler steel plates usually used to build domestic boiler, industrial boiler, and power station boiler. Boiler steel plates have the good strength .The boiler and boiler parts which made from this kind of boiler steel plates can bare certain stress in the long term at  higher temperatures.   Boiler steel plates have the  good toughness and low effectiveness of the tendency in the use of bending, roll edges and other cold deformation after processing, storage and use after a long time still maintaining good toughness, and good weldability and resistance to intergranular corrosion.
The main steel grade of boiler steel plates are : 
Material #: 1.0345, 1.0425, 1.0481, 1.0473, 1.5415, 1.7335, 1.738, 1.7383.

EN 10028-2: P235GH, P265GH, P295GH, P355GH, 16Mo3, 13CrMo4-5, 10CrMo9-10, 11CrMo9-10.

DIN 17155 : HI, HII, 17 Mn 4, 19 Mn 6, 15 Mo 3, 13CrMo 4 4, 10CrMo 9 10.

NFA 36-205 : A 37 CP, A 42 CP, A 48 CP, A 52 CP, 15 D3, 15 CD 4-05, 10 CD 9.10. 

UNI 5869: Fe 360 - 1KW, Fe 410 - 1KW, Fe 460 - 1KW, Fe 510 - 1KW, 15 Mo 3, 14 CrMo 4/5, 12 CrMO 9/10

BS 1501: 161 Gr. 360 / 164 Gr. 360 ,  161 Gr. 400 / 164 Gr. 400 / 224 Gr. 400 ,   224 Gr. 490,  1503 - 243 B,  620 Gr. 27,  622 Gr. 31 

UNE 36087: A 37 RCI, A 42 RCI, A 47 RCI, A 52 RCI, 16 Mo 3, 14 CrMo 4/5, 12 CrMo 9/10

ASTM: A 285 Gr. C  ,  A 414 Gr. C,  A 516 Gr. 55,  A 414 Gr. E,   A 516 Gr. 60,  A 414 Gr. F,  A 516 Gr. 65,  A 414 Gr. G, A 204 Gr. B/C,  A 387 Gr. 12,  A 387 Gr. 22 . 

JIS G3115: SPV 24, SPV 32, SPV 36. 

	Material #
	EN 10028-2
	DIN 17155
	NFA 36-205
	UNI 5869
	BS 1501
	UNE 36087
	ASTM
	JIS G3115

	1.0345
	P235GH
	HI
	A 37 CP
	Fe 360 - 1KW
	161 Gr. 360 / 164 Gr. 360
	A 37 RCI
	A 285 Gr. C-A 414 Gr. C / A 516 Gr. 55
	SPV 24

	1.0425
	P265GH
	HII
	A 42 CP
	Fe 410 - 1KW
	161 Gr. 400 / 164 Gr. 400 / 224 Gr. 400
	A 42 RCI
	A 414 Gr. E / A 516 Gr. 60
	

	1.0481
	P295GH
	17 Mn 4
	A 48 CP
	Fe 460 - 1KW
	224 Gr. 490
	A 47 RCI
	A 414 Gr. F / A 516 Gr. 65
	SPV 32

	1.0473
	P355GH
	19 Mn 6
	A 52 CP
	Fe 510 - 1KW
	
	A 52 RCI
	A 414 Gr. G
	SPV 36

	1.5415
	16Mo3
	15 Mo 3
	15 D3
	15 Mo 3
	1503 - 243 B
	16 Mo 3
	A 204 Gr. B
	

	1.7335
	13CrMo4-5
	13CrMo 4 4
	15 CD 4-05
	14 CrMo 4.5
	620 Gr. 27
	14 CrMo 4.5
	A 387 Gr. 12
	

	1.738
	10CrMo9-10
	10CrMo 9 10
	10 CD 9.10
	
	622 Gr. 31
	
	A 387 Gr. 22
	

	1.7383
	11CrMo9-10
	
	
	12 CrMO 9.10
	
	12 CrMo 9.10
	
	


[image: image2.jpg]Bebon steel —— The most professional exporter on shipbuilding steel
Tel:0086-371-86151527                                                      E-mail:bb@bebonchina.com                                                                     

Fax:0086-371-86011881       MSN:bebonchina@hotmail.com                                 Skype:bebonchina                                                 

Web:http://www.shipbuilding-steel.com       http://www.bebonchina.com  http://www.steel-plate-sheet.com                                   

Oficina Dir: 2801 B# Intermational enteprise center Zhengzhou city in China.


